| Name | Security Level |
| --- | --- |
| BELLSING® BAS Hydrid module | Open |
| Version | A Total of 5 Pages |
| REV B |  |

BAS Hydrid module

BVA305C00060

Product Data Sheet

| Prepared by: | Shane Wu |  | Date: | 2022-06-01 |
| --- | --- | --- | --- | --- |
| Auditor: | Nick Zheng |  | Date: | 2022-06-01 |
| Approver: | Nick Zheng |  | Date: | 2022-06-01 |

Shenzhen Bellsing Acoustic Tech. Co., Ltd.

BVA305C00060 BAS Hydrid module

# Description

## The BAS（Best Audio Speaker） Hybrid module is made up of a high performance Φ6mm dynamic speaker and a BTC305 BA receiver. This transducer is recommaned to be used in consumer electronics, especially TWS products.

| Name: | BAS Hydrid module |
| --- | --- |
| Model Number | BVA305C00060 REV B |

# Construction Specification

## Weight:  0.55 g

## Mechanical dimension：please refer to the drawing below

*A positive voltage applied to the negative terminal () will result in an increase in pressure at the sound outlet.

# Features & Benefits

## Perfect package size for TWS applications

## Outstanding efficiency

## Substantial maximum output

- Full range receiver, optimized low frequency performance

- Hybrid driver, dynamic speaker as woofer and BA receiver as tweeter.

# Electro-acoustic Specifications

## Test condition

## Acoustic tubing: tubless

## Acoutic coupler: IEC711 coupler

## Constant voltage drive: 0.179V RMS

## Environment: 23℃, RH 50%

## Acoustic Parameter I

| Test Item |  | Nominal | Tolerance | Comments |
| --- | --- | --- | --- | --- |
| Sensitivity(dB) | @20Hz | 123.0 | +/- 3.0 |  |
|  | @200Hz | 114.0 | +/- 3.0 |  |
|  | @500Hz | 109.0 | +/- 3.0 |  |
|  | @1000Hz | 109.5 | +/- 3.0 |  |
| Peak1 | Frequency (Hz) | 5000 | +/- 350 |  |
|  | SPL(dB) | 119.5 | +/- 3.0 |  |
| Peak2 | Frequency (Hz) | 9500 | +/- 1000 |  |
|  | SPL(dB) | 119.5 | +/- 3.0 |  |
| Maximum output@Peak frequency (dB) |  | N/A | N/A | THD<10% |

## Acoustic Parameter II

| Test Item |  | Nominal | Comments |
| --- | --- | --- | --- |
| THD(%) | @50-5000Hz | ＜5.0 |  |

## Electric Parameter

| Test Item | Nominal | Tolerance | Comments |
| --- | --- | --- | --- |
| Impedance @500Hz | 30.4 | +/- 15% |  |
| Impedance @1000Hz | 27.3 | +/- 15% |  |
| DCR @20℃ Speaker | 32.0 | +/- 15% |  |
| DCR @20℃ Receiver | 12.0 | +/- 10% |  |

## Nominal Frequency Response Curve

# Package and Storage Condition

## Package: PVA bubble pack, 1 pcs per bubble, 144 pcs per pack

## Storage temperature range: -40℃ ~ 60℃

## Storage humidity range: <60%

# Reliable Test

| ## No | ## Item | ## Test Conditions |
| --- | --- | --- |
| 1 | Operation Test | Input power 5mW sine Wave Input 0.25v Must Be Normal |
| 2 | #### Heat Test | 60℃ + 3℃. 48 Hours After Test Meet Item 1 |
| 3 | #### Heat Test | 60℃ + 2℃ 20%-50% 48 Hours After Test Meet Item 1 |
| 4 | #### Humidity Test | 60℃ + 2℃ 90-95%. 24 Hours After Test Meet Item 1 |
| 5 | #### Load Test | White Noise 2mW 96 Hours After Test Meet Item 1 |
| 6 | #### Cold Test | -25℃ + 3℃ 45-85% 48Hours After Test Meet Item 1 |
| 7 | #### Vibration Test | F=10~55Hz Amplitude:1.5mm Test Direction is X, Y,Z 2H/each. |
| 8 | #### Dropping Test | Effective height 1M Slope 75º+2º |
| 9 | #### Insulation Resistance | More than 1MΩ +dc 100V+10% |
| 10 | #### Cold hot circulation | 60℃+1℃/0.5h→20℃/0.5h→-20℃/0.5h→20/0.5h 10cycles and then on 20℃ 3 hours |
| 11 | #### life test | 2mW white noise was input by audio signal generator for 48 hours, and there was no adverse phenomenon during the test |