| Name | Security Level |
| --- | --- |
| BELLSINGTM BPS Receiver | Open |
| Version | A Total of 4 Pages |
| REV A |  |

BPS1010FBA08

Product Data Sheet

| Prepared by: | Jack zhao |  | Date: | 2022/9/28 |
| --- | --- | --- | --- | --- |
| Auditor: | Nick zheng |  | Date: | 2022/9/28 |
| Approver: |  |  | Date: |  |

Shenzhen Bellsing Acoustic Tech. Co., Ltd.

BPS1010FBA08 BPS receiver

# Description

## BPS receiver is a specially designed acoustic transducer, using dynamic speaker tehnology but having a smaller size like a BA receiver.It has a full range balanced performance and is a good solution for In-ear monitor devices and other commercial hearing products.

| ## Name: | ## BPS Receive |
| --- | --- |
| ## Part Number: | ## BPS1010FBA08 REV A |

# Construction Specification

## Weight:  1.08 g

## Mechanical dimension：please refer to the drawing below

*A positive voltage applied to the negative terminal () will result in an increase in pressure at the sound outlet.

# Features & Benefits

## Perfect package size for in-ear headsets, TWS and VR/AR devices.

## Planarized diaphragm and coil structure provides substantial sound pressure level.

## Full-range response through out audio frequencies.

# Electro-acoustic Specifications

## Test condition

## Acoutic coupler: IEC711 coupler

## Constant voltage drive: 0.126V RMS

## Environment: 23℃, RH 50%

## Acoustic Parameter I

| Test Item |  | Nominal | Tolerance | Comments |
| --- | --- | --- | --- | --- |
| Sensitivity(dB) | @20Hz | 127.5 | +/- 3.0 |  |
|  | @500Hz | 131.5 | +/- 3.0 |  |
|  | @1000Hz | 127 | +/- 3.0 |  |
| Peak1 | Frequency (Hz) | 600 | +/- 300 |  |
|  | SPL(dB) | 132 | +/- 3.0 |  |
| Valley1 | Frequency (Hz) | 3500 | +/- 500 |  |
|  | SPL(dB) | 96 | Minimum |  |
| Peak2 | Frequency (Hz) | 4500 | +/- 500 |  |
|  | SPL(dB) | 116.5 | +/- 3.0 |  |

## Acoustic Parameter II

| Test Item |  | Nominal | Comments |
| --- | --- | --- | --- |
| THD(%) | 20-5000hz | <5.0 |  |

## Electric Parameter

| Test Item |  | Nominal | Tolerance | Comments |
| --- | --- | --- | --- | --- |
|  | @500HZ | 8 | +/- 15% |  |
| Impedance | @1000Hz | 8 | +/- 15% |  |
| DCR(Ω) |  | 8 | +/- 10% |  |

## Nominal Frequency Response Curve

# Package and Storage Condition

## Package: PET bubble pack, 1 pcs per bubble, 60 pcs per pack

## Storage temperature range: -40℃ ~ 60℃

## Storage humidity range: <60%